- Opel ADAM: pregled tehničkih podataka

| Engines | 1.2 / 1.2 ecoFLEX | 1.4 / 1.4 ecoFLEX | 1.4 / 1.4 ecoFLEX |
| --- | --- | --- | --- |
| - Emission classification | EURO5 | EURO5 | EURO5 |
| - Fuel | Super | Super | Super |
| - Number of cylinders | 4 | 4 | 4 |
| - Displacement cm3 | 1229 | 1398 | 1398 |
| - Bore / stroke in mm | 73,4 / 72,6 | 73,4 / 82,6 | 73,4 / 82,6 |
| - Max. output in kW (hp) / at rpm | 51 (70) / 5600 | 64 (87) / 6000 | 74 (100) / 6000 |
| - Max. torque in Nm / at rpm | 115 / 4000 | 130 / 4000 | 130 / 4000 |
| Compression | 10,5 : 1 | 10,5 : 1 | 10,5 : 1 |
| Final drive ratio | 3,74 | 3,94 | 3,94 |
| Fuel tank capacity in l | 38 | 38 | 38 |
| - Weight and axle loads in kg |  |  |  |
| - Curb weight incl. driver (according to 70/156/EEC) | 1086 | 1120 | 1120 |
| - Permissible gross vehicle weight | 1455 | 1465 | 1465 |
| - Payload | 369 | 345 | 345 |
| - Permissible axle load, front | 805 | 805 | 805 |
| Permissible axle load, rear | 680 | 690 | 690 |
| - Permissible roof load 1) | 50 | 50 | 50 |

| - 5-speed manual | - Maximum speed - in km/h | - Acceleration - from 0 – 100 km/h in s | Elasticity 80-120 km/h in 5th gear in seconds | Fuel consumption (l/100 km) |  |  | - CO2 emissions in g/km (combined) | Efficiency rating |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | - Urban | - Extra-urban | - Combined |  |  |
| 1.2 (51 kW) | 165 | 14,9 | 26,8 | 7,1 | 4,2 | 5,3 | 124 | D |
| 1.2 ecoFLEX (51 kW) with Start/Stop | 165 | 14,9 | 26,8 | 6,5 | 4,2 | 5,0 | 118 | C |
| 1.4 (64 kW) | 176 | 12,5 | 17,8 | 7,3 | 4,4 | 5,5 | 129 | D |
| 1.4 ecoFLEX (64 kW) with Start/Stop | 176 | 12,5 | 17,8 | 6,6 | 4,2 | 5,1 | 119 | C |
| 1.4 (74 kW) | 185 | 11,5 | 17,8 | 7,3 | 4,4 | 5,5 | 129 | D |
| 1.4 ecoFLEX (74 kW) with Start/Stop | 185 | 11,5 | 17,8 | 6,6 | 4,2 | 5,1 | 119 | C |

All data are subject to alteration and refer to the European base model with standard equipment. Fuel consumption data and CO2 emissions data are determined according to regulation 2007/715/EC, taking into consideration the vehicle curb weight, as stipulated by the regulation. Additional equipment can lead to slightly higher fuel consumption and therefore CO2 exhaust emissions than the declared values. In addition, they can raise the vehicle’s curb weight and in some cases also increase the gross vehicle weight, the maximum allowable axle loads and respectively reduce the permitted trailer load. Therefore the maximum speed may be decreased while acceleration time can be increased. The published performance figures are possible with the vehicle’s curb weight excluding driver plus a 200 kilogram load allowance.

| Dimensions and Weights |  |
| --- | --- |
| - Vehicle dimensions in mm |  |
| - Length | 3698 |
| - Width including / excluding door mirrors | 1966 / 1720 |
| - Height (at curb weight) | 1484 |
| - Wheelbase | 2311 |
| - Track, front | 1472 |
| - Track, rear | 1464 |
| - Turning circle in m |  |
| - Wall-to-wall | 10,17 - 11,06 |
| - Curb-to-curb | 9,80 - 10,74 |
| - Luggage compartment dimensions in mm |  |
| - Floor length to rear seat backs | 462 |
| - Floor length with rear seat backs folded forward | 1041 |
| - Width measured at wheel housing | 954 |
| - Max. width | 963 |
| - Height of sill | 808 |
| - Max. aperture height | 544 |
| - Max. aperture width measured at waistline | 853 |
| - Luggage compartment in liters (according to ECIE) |  |
| - Luggage compartment only | 170 |
| - With rear seat backs folded forward up to front seat backs | 484 |
| - With rear seat backs folded forward up to ceiling | 663 |
| Reduction of luggage compartment with |  |
| - Infinity Sound System | 75 |

All data are subject to alteration and refer to the European base model with standard equipment.  In some trim levels, the curb weight and in some cases the permissible axle loads can increase the permissible gross vehicle weight or reduce the payload.

1) Under consideration of gross vehicle weight. For safety reasons it is recommended not to drive over 120 km/h with a roof load.